# Design Document : Neural Accelerator for RISCV

## Abstract:

The Neural and ML Accelerator is a custom ASIC accelerator designed for low-power, high-throughput convolutional neural network (CNN) inference at the edge. It features a 32-bit RISC-V management core, a systolic array for matrix multiplication, and dedicated on-chip memory to minimize off-chip DRAM traffic.

In addition to the matrix acceleration engine, the architecture integrates multiple hardware modules commonly required during neural network forward-pass processing, such as activation, pooling, normalization, accumulation units. The accelerator follows a modular instruction-set-based architecture and pipeline inspired by the principles of RISC-V, where different compute modules are selectively multiplexed and controlled through configurable instructions.

As a dedicated edge-processing accelerator chip, the design is optimized to execute machine learning workloads with significantly lower latency and power consumption compared to general-purpose processors. By performing computations locally on-device, the accelerator enables faster real-time inference for applications such as computer vision, autonomous systems, IoT devices, robotics, and intelligent sensing without relying heavily on cloud connectivity.

Key Features

- 32-bit RISC-V inspired instruction architecture

- Modular opcode-driven execution engine

- 3-stage pipelined datapath

- Dedicated 8×8 systolic array

- INT8 computation with 32-bit accumulation

- Edge-AI optimized low-power architecture

- Support for neural-network activation and quantization modules

## Architecture:

- Instruction Architecture

The instruction format contains:

| Field | Purpose |
| --- | --- |
| rd/wr | Read or write operation |
| addr_A | Base address for Matrix A |
| addr_B | Base address for Matrix B |
| opcode | Select operation |
| clk | Clock signal |
| rst | Reset signal |
| data_in | Input data during write mode |

Control and Interface Signals

The accelerator is controlled using a simple instruction-based interface containing the following signals:

- rd/wr bit

  - Controls load/store operation

  - 0 → Read/Execute mode

  - 1 → Write mode

- 8-bit Register A Address (addr_A)

  - Selects starting address of Matrix A registers

- 8-bit Register B Address (addr_B)

  - Selects starting address of Matrix B registers

- 4-bit Instruction Opcode

  - Supports 16 unique instructions

  - Selects the operation module to execute

- clk signal

  - System clock for synchronous operation

- rst signal

  - Resets registers and internal states

- 8-bit Data Input (data_in)

  - Used during write/load operation to store values into registers

- 32-bit Output (cout)

  - Outputs final computation result from accumulator registers

Working Principle

- The opcode selects the required compute module

- A multiplexer routes inputs and outputs to the selected execution block

- Only the required hardware module becomes active for execution

- Memory Architecture : Register Bank Structure

Register Bank A :Stores a 4×4 Matrix A

Register Bank B : Stores a 4×4 Matrix B

The matrices are formed using consecutive register locations.

Example:

- If addr_A = X

  - Matrix element A[0][0] → Register X

  - A[0][1] → Register X+1

  - A[0][2] → Register X+2

  - and so on...

The same method is used for Matrix B.

This matrix construction happens during the Instruction Decode Stage.

Internal Computation Registers

Padded Matrix Registers

This is a new approach to build a padded matrix to reduce the design complexity by constructing a padded matrix for better timing synchronization.

For internal processing:

- Two separate 8×8 register arrays are used

- Each register is 8 bits

- Used for:

  - Matrix padding

  - Parallel computation

  - Alignment with systolic processing structure

These internal arrays temporarily store processed versions of Matrix A and Matrix B before execution.

8×8 Accumulator Array

- Stores intermediate multiplication and accumulation results

- Each accumulator register is 32 bits wide

- Prevents overflow during large computations

The final 4×4 result matrix is connected to the cout output line through wired connections from the accumulator registers.

## Microarchitecture ,Datapath ,Control Unit:

The proposed Neural and ML Accelerator follows a modular microarchitecture consisting of a control unit, Datapath, execution engine, register banks, and a 3-stage pipelined processing structure. The architecture is designed for efficient neural-network feedforward computation and systolic-array-based parallel execution.

Overall FSM Control

This FSM enables synchronization between the processor and accelerator while providing execution status monitoring. The accelerator operates using a finite state machine (FSM) containing three primary states:

- IDLE : Default waiting state , Waits for instruction and control signals

- COMPUTE :  Busy flag is asserted (busy = 1), Instruction execution and systolic computation occur in this stage

- DONE : Indicates completion of execution, Processor can read the output/result status from the accelerator

3-Stage Pipeline Architecture

The Datapath is organized into a 3-stage pipeline:

Stage 1 — Data Decode Stage

In the decode stage, register-bank access is performed.

Operations performed in this stage include:

- Reading Register Bank A and Register Bank B

- Constructing matrices using consecutive memory locations

For matrix operations:

- mem[addr_A] → A[0][0]

- mem[addr_A + 1] → A[0][1]

- and similarly for the remaining matrix elements

The same mechanism is used for Matrix B.

For operations requiring only a single register operand:

- Only A[0][0] or the first required register value is taken in the execution stage

For array-based operations:

- The entire row/vector (A[0]) is extracted in the execution stage

The decode stage also forwards opcode ,matrix data ,addresses ,control signals ,clock/reset signals to the execution pipeline stage.

Stage 2 — Execute Stage

The execute stage contains the main computation engine and instruction execution modules. The instruction opcode is routed through a multiplexer structure implemented using the case construct in Verilog. Depending on the opcode, the corresponding execution module becomes active. The architecture currently supports 16 dedicated instruction modules for:

- matrix operations

- neural-network processing operations

- systolic-array-based execution

Two clock domains are used:

- clk : Used for systolic data movement and PE shifting operations

- bigclk : Used for pipeline-level instruction execution, Generated internally using a counter

Since the systolic array requires 8 clock cycles to complete computation, bigclk is generated using: (bigclk = ~bigclk ) after every 8 clk pulses through counter-based clock division logic.  This allows synchronization between systolic computation latency and instruction execution timing.

Stage 3 — Write-Back Stage

The final stage performs memory update and output write-back operations. Functions of this stage include:

- Writing c_out back into: register banks ,intermediate registers

- Handling external write operations :

If the write signal is enabled: mem[addr_A] = input_data;  can be performed directly to store external data into the register bank.

The write-back stage also generates: completion signal ,done status ,output forwarding

for processor-level synchronization.

Datapath Structure

The datapath consists of:

- Register Bank A

- Register Bank B

- Matrix construction logic

- Instruction decoder

- Multiplexer-based execution selector

- 16 execution modules

- Systolic computation array

- Accumulator array

- Write-back routing logic

The datapath is optimized for:

- parallel execution

- low-latency matrix computation

- reduced memory access overhead

- efficient edge-AI inference

Current Implementation Status

Currently:

- The execution block and instruction modules are fully implemented

- Decode logic, FSM control, and write-back stages are architecturally defined and partially ideated

The modular pipeline-based design allows scalability and future integration of additional ML operations while maintaining compatibility with the instruction-driven accelerator architecture inspired by RISC-V principles.

## Execution block:

The execution block forms the main computation engine of the accelerator and is responsible for performing neural-network and machine-learning operations selected through the instruction opcode. The module follows a modular execution architecture where each operation is implemented as an independent hardware block and selected using opcode-based multiplexing. The execute stage operates as the second stage of the 3-stage pipeline and receives decoded data, matrix inputs, control signals, and register operands from the decode stage.

The execution block uses two clock domains. Since the systolic array requires 8 clock cycles for computation completion, bigclk is generated internally using a counter-driven clock division mechanism.

Inputs to Execute Block

- reg_a and reg_b for scalar operations

- matrix_a and matrix_b for matrix operations

- opcode for instruction selection

- addr_a and addr_b for pipeline forwarding

- clk, bigclk, and rst control signals

Outputs

- cout : 32-bit execution result

- Forwarded pipeline control signals:

  - addr_a_next

  - addr_b_next

  - rd_wr_bar_next

Execution Modules

The execution engine currently contains multiple dedicated hardware modules commonly required during neural-network forward-pass computation. Single register input modules are:

ReLU Module

- Implements Rectified Linear Unit activation

- Outputs zero for negative inputs

MAC Module

- Performs signed INT8 multiply-and-accumulate operation

- Used for neural-network accumulation and convolution computation

Dot Product Module

- Computes vector dot product using parallel MAC units

- Used in matrix multiplication and feedforward operations

Popcount Module

- Counts number of active bits in an 8-bit input

- Useful in binary neural network computation

Clip Module

- Restricts outputs within programmable minimum and maximum ranges

- Used for quantized inference control

Scaling Module

- Performs fixed-point scaling and shift operations

- Used during quantization and normalization

Sigmoid Approximation Module

- Implements low-cost sigmoid activation approximation

- Optimized for hardware-efficient inference

Tanh Approximation Module

- Provides approximate hyperbolic tangent activation function

Max Finder Module

- Finds maximum value and corresponding index from multiple inputs

- Useful for classification and pooling operations

BNN Module

- Binarized Neural Networks (BNNs) are highly efficient neural networks where weights and activations are constrained to binary values, typically  -1 or +1. By replacing standard 32-bit floating-point operations with XNOR and bit-count operations, BNNs drastically reduce memory usage and increase inference speed, making them ideal for edge devices and IoT applications

Matrix Multiplication Module

The accelerator includes a dedicated 4×4 matrix multiplication module designed for neural-network and feedforward computations. The module accepts two signed INT8 matrices and generates a 32-bit output matrix. It is controlled using clk, rst, valid, and done signals. Input matrices are latched and Matrix B is transposed for efficient access. Parallel dot_product_4 modules compute matrix elements simultaneously. Results are written into the output matrix and the done signal is asserted.

Dot Product and MAC Modules

The dot_product_4 module computes vector dot products using four parallel int8_mac units. Each MAC unit performs signed INT8 multiplication and accumulation to generate 32-bit outputs. These modules form the basic computation blocks for matrix multiplication and neural-network arithmetic operations.

Systolic Array Module

The accelerator contains an 8×8 systolic array optimized for parallel matrix computation. Internally, 4×4 input matrices are padded into 8×8 arrays for synchronized systolic data movement and timing alignment.

The systolic array consists of:

- Padding logic (a_pad, b_pad)

- Processing Element (PE) grid

- 8×8 accumulator array

- Dataflow routing network – Facilitated by the PE logic

Processing Element (PE)

The Processing Element is the core compute unit of the systolic array.

Each PE performs:

- Input data latching

- Multiply-and-Accumulate (MAC) operation

- Horizontal and vertical data forwarding

Partial sums are stored in accumulator registers and propagated through the array during systolic execution.

Accumulator Array : An internal 8×8 accumulator array stores intermediate 32-bit partial sums generated during matrix computation. The final active 4×4 results are forwarded to the output matrix c_out.

Opcode-Based Execution

Instruction execution is controlled using a case-based opcode selection structure implemented in Verilog. Based on the 4-bit opcode, the corresponding execution module is activated and its output is routed to the final 32-bit output register (cout). This modular architecture enables scalability and easy integration of additional ML operations in future versions of the accelerator.

Potential Applications

- Edge AI devices

- Computer vision systems

- Smart surveillance

- Robotics and autonomous systems

- IoT-based intelligent sensing

- Wearable AI devices

- Embedded neural-network inference

- Real-time signal and image processing

The systolic-array-based architecture enables efficient matrix computation while minimizing memory-access overhead, making the design suitable for low-power ASIC implementations.

## Timing diagram for execution block:

1. The `execute_top` module acts as the centralized execution stage of the accelerator pipeline, interfacing fetch/decode outputs with arithmetic, neural-network, and matrix-computation hardware blocks through opcode-driven control.

2. The architecture integrates multiple AI-oriented compute units including MAC, dot-product, activation functions (ReLU, Sigmoid, Tanh), BNN operations, scaling, clipping, and ArgMax within a unified parallel execution framework.

3. Dedicated hardware accelerators for conventional 4×4 matrix multiplication and systolic-array-based multiplication are selectively enabled using control signals (`valid_mat`, `en_sys`) to support heterogeneous computation modes.

4. A hierarchical timing scheme is implemented using `clk` for internal accelerator operation and a slower `bigclk` for pipeline-stage synchronization, enabling controlled execution latency and simplified inter-stage data transfer.

5. The design supports pipelined processor integration by forwarding register addresses and control signals across stages, allowing seamless connection between fetch/decode, execute, and writeback modules in a modular AI accelerator datapath.

* The accelerator uses a dual-clock timing scheme where `clk` controls internal computations and `bigclk` synchronizes pipeline-stage execution.

* Input matrices are diagonally skewed through `a_pad` and `b_pad` registers to ensure synchronized operand arrival across the systolic array.

* During each clock cycle, operands propagate horizontally and vertically between Processing Elements (PEs) using `a_wire` and `b_wire` interconnects.

* Partial sums are accumulated recursively through the `accumulator → acc_next` feedback path, enabling pipelined MAC computation.

* Control signals such as `valid_mat`, `en_sys`, `done_mat`, and `done_sys` manage accelerator enable, pipeline flushing, and output-valid timing before writeback.

mat_A=[ 0 1 2 3

1 2 3 4

2 3 4 5

5 6 7 8]

Mat_B= [1 2 3 4

5 6 7 8

9 10 11 12

13 14 15 16]

For different opcodes:

For matrix_mult:

- The matrix_mult_4x4 module implements a parallel 4×4 matrix multiplication architecture using 16 concurrent dot_product_4 computation units for high throughput operation.

- An FSM-based control path manages data loading, transpose generation, computation triggering, result storage, and completion signaling across multiple pipeline stages.

- Matrix B is internally transposed to improve row-column data access efficiency, enabling simultaneous dot-product evaluation for all output elements.

- The design uses staged valid-control signals (stage1_done, stage2_valid, done) to synchronize pipeline execution and ensure deterministic timing behavior during hardware acceleration.

Systolic :

The accelerator implements a diagonally skewed data-injection scheme using padded shift registers (a_pad and b_pad) to guarantee cycle-accurate operand alignment across the systolic fabric without requiring complex control routing.The accelerator uses a scalable 8×8 systolic-array architecture with diagonally skewed data injection, pipelined MAC-based distributed accumulation, and operand forwarding for temporal data reuse, while a lightweight counter-driven timing scheme handles synchronization and result generation without centralized FSM control. Matrix tilling id done by processor

A lightweight cycle-driven control mechanism using the internal counter c manages operand streaming, synchronization, pipeline flushing, and result-valid timing, eliminating the need for a centralized FSM-based scheduler.